ITX610B(R100)

Product Features:

◆ Intel 12th generation Alder Lake-S series/13th generation Raptor Lake-s series, LGA1700, TDP 65W; H610/Q670;

◆ 2*DDR5 SO-DIMMs, up to 64GB;

◆ 2*Intel Network i219-LM + i226-V, 2*HDMI, 2*DP, DP/LVDS

◆ M-Key 2280，E-Key，PCIe_X16

◆ Power supply: DC 12-24V Size: 170x170 mm

Product specification

| Processor system | Intel 12th generation Alder Lake-S series/13th generation Raptor Lake-S series, LGA1700, TDP 65W |
| --- | --- |
|  | EFI BIOS |
| Memory | 2*DDR5 SO-DIMM 4800MHz, up to 64GB |
| Storage | 1*M.2 M-Key 2280 (NVMe PCIe4.0_x4 default / optional SATA3.0 protocol) storage interface |
|  | 2*SATA3.0 interface |
| Display | 2*HDMI2.1 interface, support 4096x2160@60Hz |
|  | 2*DP1.4, support 7680x4320@60Hz |
|  | eDP/LVDS |
| Board edge I/O interface | DC JACK, 2*USB2.0 |
|  | 2*HDMI/DP |
|  | 2*(LAN + 2*USB3.2), 2-hole audio input and output |
| Extended Interface/Function | TPM2.0 optional, N/A by default |
|  | 1*PCIe_X16 (PCIe4.0_16x protocol) |
|  | 1*M.2 E-Key (PCIe3.0/USB2.0 protocol, support WIFI/BT module) |
|  | 4*COM pin header, 2x5Pin, pitch 2.54m; COM1/2 can choose RS232/485, COM3/4 can be RS232 |
|  | 2 groups of 4 USB2.0 pin headers, with a pitch of 2.54mm (one USB2.0 and BT share LAY) |
|  | 8*GPIO header, 2x5Pin, pitch 2.54mm |
|  | 1*4Pin PWM CPU fan, 1*4Pin system fan |
| Power Supply | DC 12-24V, above 120W |
| Working Environment | Working temperature: -20℃ ~ +60℃; Working humidity: 5% ~ 90% |
|  | Storage temperature: -40℃ ~ +85℃; storage humidity: 5% ~ 90% |
| Operating System | Windows10, Windows11, Linux |
| Size | 170x170mm |
| weight | about 400g |

ITX610B(R100)

产品尺寸                                                  单位：mm

Ordering Information

| Model | Options | Descriptions |
| --- | --- | --- |
| ITX610B | External TPM2.0 | External TPM2.0 optional |
|  | Serial port COM1/2 optional 232/485 | RS232/RS485 optional |
|  | 1 set of USB2.0 pins | One set of USB2.0 pins and BT are optional |
|  | chipset | H610/Q670 optional |
|  | M.2 M-Key | PCIe4.0 x4/SATA3.0 protocol optional |